Instituto

Politécnico

Nacional

Escuela Superior de Cómputo

3_4 Managing Device Configurations

Materia:

Administración de servicios en red

Grupo:

4CV13

Profesor:

Henestrosa Carrasco Leticia

Integrantes: (Equipo 1)

Arévalo Andrade Miguel Ángel

Castro Cruces Jorge Eduardo

López Mares Irene Elizabeth

Pedroza García Rodolfo

Fecha:

Jueves, 26 de mayo de 2022

## Activity 11.5.2:
Managing Device Configurations

NOTE TO USER: This activity is a variation of lab 11.5.2 This Packet Tracer Activity is not a companion for the above named lab. The instructions for completing this lab are found within this lab.

### Addressing Table

| Device | Interface | IP Address | Subnet Mask | Enable Secret Password | VTY/ Console Passwords |
| --- | --- | --- | --- | --- | --- |
| ROUTER1 | Fa0/0 | 192.168.1.1 | 255.255.255.0 | class | cisco |

### Learning Objectives

- Perform basic router configurations

- Backup a copy of a router configuration file

- Reload the backup configuration file from a TFTP server into RAM on a router

- Save the new running-config to NVRAM

### Introduction:

In this lab, students will configure common settings on a Cisco Router, save the configuration to a TFTP server, then restore the configuration from a TFTP server.

### Task 1: Configure ROUTER1

#### Step 1: Basic Configurations on ROUTER1

Using the table at the beginning of the lab, configure the router hostname.

Configure the FastEthernet interface and configure it with a description.

Protect access to the console port using cisco as the password.

Configure the router with an encrypted enable password of class.

Restrict remote access to the router using a password of cisco.

Configure a banner that will warn unauthorized personnel that access is prohibited.

Verify the router’s configurations by performing the show running-config command on the router.

If not correct, fix any configuration errors and re-verify. Save the configurations to NVRAM.

#### At the end of this task your completion rate should be 62%.

### Task 2: Configure the TFTP Server

#### Step 1: Configure the TFTP server

Using the following information apply a layer 3 address and default gateway to the TFTP server:

- IP Address: 192.168.1.2

- Subnet mask: 255.255.255.0

- Default Gateway: 192.168.1.1

#### Step 2: Verify connectivity

Ping the TFTP server from ROUTER1. If the ping fails, review the TFTP and router configurations to resolve the problem.

#### At the end of this task your completion rate should be 100%.

### Task 3: Backup the startup-config to the TFTP Server

#### Step 1: Copy configurations

Use the Copy Start TFTP command on ROUTER1. Enter the TFTP IP address as the address of the remote host; and leave all other questions as; the default (Press Enter)

#### At the end of this task your completion rate should be 100%.

### Task 4: Verify Configuration Transfer to the TFTP Server

#### Step 1: Verify TFTP transfer

First, click on the TFTP server. Next, click on the Config Tab. Then, click on the TFTP tab. Verify that the ROUTER1-config file is listed (should be at the bottom of the list)

#### At the end of this task your completion rate should be 100%.

Conclusiones:

Arévalo Andrade Miguel Ángel:

• Se realizaron configuraciones básicas del enrutador

• Se hizo la copia de seguridad de una copia de un archivo de configuración del enrutador

• Se volvió a cargar el archivo de configuración de copia de seguridad desde un servidor TFTP en la RAM de un enrutador

• Se guardó la nueva configuración en ejecución en NVRAM

Castro Cruces Jorge Eduardo:

• Se realizaron configuraciones básicas del enrutador

• Se hizo la copia de seguridad de una copia de un archivo de configuración del enrutador

• Se volvió a cargar el archivo de configuración de copia de seguridad desde un servidor TFTP en la RAM de un enrutador

• Se guardó la nueva configuración en ejecución en NVRAM

López Mares Irene Elizabeth:

• Se realizaron configuraciones básicas del enrutador

• Se hizo la copia de seguridad de una copia de un archivo de configuración del enrutador

• Se volvió a cargar el archivo de configuración de copia de seguridad desde un servidor TFTP en la RAM de un enrutador

• Se guardó la nueva configuración en ejecución en NVRAM

Pedroza García Rodolfo:

• Se realizaron configuraciones básicas del enrutador

• Se hizo la copia de seguridad de una copia de un archivo de configuración del enrutador

• Se volvió a cargar el archivo de configuración de copia de seguridad desde un servidor TFTP en la RAM de un enrutador

• Se guardó la nueva configuración en ejecución en NVRAM